IOP Analytics & Ruleset Pack Architecture

- IOP as a platform

The Intelligent Operations Platform (IOP) is intended to serve multiple industries, each with:

- Different data sources (Camera, GPS, IoT sensors, PLC, SCADA, etc.)

- Different operational intelligence needs

- Different workflows

- Different domain-specific optimization apps

Because of this, IOP must offer a generic, reusable, parameter-driven analytics framework, capable of expressing any operation through:

- A standard set of low-level operators

- A declarative operation template (pack metadata)

This ensures:

- Minimal code inside edge runtime

- Scalable multi-industry expansion

- Easy release management

- Third-party extension support

- Lightweight footprint on edge devices

So, basically IOP should function like an analytics engine (+ rule engine) powered by reusable building blocks, with industry packs acting as recipes, not as a point solution.

- Standardized Dual-Lane Pipeline + Modular Operators

IOP requires a two-lane pipeline design, each powered by reusable operators:

# Analytics lane (batch/window-based processing)

Purpose:

- Operational inefficiencies

- Trend analysis

- Long-window insights

- Time-series modelling

- Predictive analytics

Typical Flow:

Ingestion → Data Resampler → InfluxDB → Analytics Processor → Events/Insights → Optional Optimizer App

Characteristics:

- Time-window based (hours, days, weeks - e.g., 5 days of data)

- Uses heavier statistical/ML models

- Aggregates across multiple streams

- Not strictly real-time

- Produces higher-level insights, not violations

Outputs:

- Insights

- Recommendations

- Trigger for optional optimization apps

# Ruleset Lane (strict real-time, immediate impact)

Purpose:

- Safety violations

- Compliance rules

- Real-time incidents

Typical Flow:

Ingestion → Data Resampler → Rule Processor → Events/Incidents → Notifications + Incident Workflow

Characteristics:

- Must be ultra-low-latency

- Strict per-frame/per-event evaluation

- ML models MUST run on the edge

- Deterministic logic (if condition then flag violation)

- Produces immediate alerts

Outputs:

- Incidents

- Trigger notification and incident workflow

- Key Principles for Pipeline Building Blocks

To prevent the platform from becoming bloated:

# Principle 1: Operators must be generic, reusable low-level primitives

These are not domain-specific, and can support any pack when parameterised.

Refer Appendix 8.1 for core data operators.

# Principle 2: Packs should be declarative, not code-driven

Each pack defines only:

- The ML models it requires

- The operator sequence (pipeline graph)

- The parameter schema

- The insights it emits

Pack updates should not require rebuilding the edge runtime.

# Principle 3: Edge Runtime Must Stay Lightweight

Only includes:

- The set of ~15 reusable operators

- A dynamic model loader

- A pipeline execution orchestrator

- A small rules engine

# Principle 4: Optimization logic belongs in optional Apps, not in the Analytics

Examples:

- Route Optimizer App (Google OR-Tools)

- Predictive Maintenance Planner App

- Energy Optimization Scheduler

The analytics engine only detects inefficiency; the optional App fixes or optimizes it.

- Release Management Benefits

Because of the reusable operator architecture:

Adding a new pack does NOT require:

- Updating Analytics Processor code

- Rebuilding the edge runtime

- Installing new operators

Updating a pack ONLY requires:

- Updating a JSON/YAML descriptor

- Adding or updating ML model artifacts

- Modifying pack metadata

- Optionally updating App logic (external component)

Edge footprint remains small and stable:

- Only ~15 core operators

- Models loaded dynamically per pack

- Ruleset lane remains unaffected

- Final Recommendations

- Standardize the entire analytics ecosystem around low-level operator reuse

  - This prevents “operator explosion”

- Enforce the Dual-Lane Architecture

  - Ruleset Lane for real-time safety

  - Analytics Lane for batch-based insights

- Packs must be declarative, not compiled

  - No pack-specific logic inside edge processor.

- Optimization Apps must remain external

  - The Analytics Processor should detect issues, not fix them.

- ML models must be downloaded on-demand

  - Each site only downloads models relevant to subscribed packs.

- Strict separation of concerns

  - Vision/Sensors → Streamers

  - Aggregation → Resampler + WindowSelector

  - Analysis → Analytics Processor

  - Real-time Evaluations → Rule Processor

  - Optimization → Optional Apps

- Tech Stack

- Python for orchestrating operators & pipeline definition

- Rust micro-libraries for heavy operators (trajectory, clustering, anomaly detection, etc.)

- PyO3 to call Rust from Python

- Pydantic / FastAPI style component for config validation

- Polars (Rust-powered DataFrame) for ultra-fast computation

- NumPy / SciPy for statistical operators

- PyTorch / ONNX Runtime for ML operators

- Conclusion

The revised architecture transforms IOP into a:

- Multi-industry, scalable analytics platform

- Easy-to-maintain edge-friendly system

- Low-latency real-time safety engine

- Flexible marketplace-driven intelligence ecosystem

- Developer-friendly system that supports adding packs without runtime changes

The provided pipeline is valid in concept, but must be implemented using standardised operators, not custom code.

- Appendix

# Core Data Operators (Shared Across All Industries)

# Ingestion & Windowing

| Operator | Purpose |
| --- | --- |
| WindowSelector | Time-based selection (sliding, tumbling) |
| JoinOperator | Combine multiple sensors/vision streams |
| FilterOperator | Threshold, conditional, event filtering |
| Aggregator | Mean, max, std, percentiles |

# Sequence & Graph Processing

| Operator | Purpose |
| --- | --- |
| TrajectoryExtractor | Turns GPS sequences → trajectories (generic) |
| GraphBuilder | Builds graph from points or OSM extract |
| PathComparer | Compute similarity, overlaps, divergence |
| ClusterOperator | K-Means, DBSCAN, HDBScan |

# Statistical/ML Operators

| Operator | Purpose |
| --- | --- |
| AnomalyDetector | Statistical or ML anomaly detection |
| ForecastOperator | Time-series future prediction |
| ClassificationOperator | Generic ML classification |
| RegressionOperator | Generic ML regression |

# Utilities

| Operator | Purpose |
| --- | --- |
| ThresholdEvaluator | Apply user-defined rule/parameter |